How to adjust the shade length for cordloop TDBU/Day&Night

- Please raise the middle rail to the top, and pull the bottom rail out from the window sill and then measure the distance from bottom of the bottom rail to the window sill (This distance need to be remembered).

- Adjust the middle rail to the convenient operation height, pull the shade out, people stand in the place between the shade and the window, then you can see the adjusting button on back of the middle rail。

3. Open the adjusting button, pull out the cord, refer to the above distance from bottom rail to the window sill and the actual demand to re-knot cords (all the cords must be retie at the same place), finally close the adjusting button